Connecting Cameo Systems Modeler to Safety Lab Aero

CATIA Magic / MagicDraw — Requirements via ReqIF, Architecture via SysML XMI

July 2026 · Safety Lab Aero

Table of Contents

Right-click and choose “Update Field” to build the table of contents.

# What this integration does

Cameo is the MBSE side of the thread rather than the RM system of record, so the integration has two lanes: requirements and traces travel by ReqIF (same loop as DOORS/Polarion), and architecture travels by SysML XMI into Safety Lab’s model importer. Together: model the architecture in Cameo, run the safety spine in Safety Lab, keep requirements in the RM tool — all meeting at the failure-condition objects.

# Lane 1 — Requirements & traces via ReqIF

Cameo’s requirements interchange (built into recent versions; historically the Cameo Requirements Modeler plugin) imports and exports ReqIF, including packages from DOORS, Polarion, Integrity and Teamcenter.

Export the FC package from Safety Lab (“📄 Export ReqIF” on the AC FHA page).

In Cameo: File → Import From → Requirements Interchange Format (ReqIF) → choose the file. The FCs appear as SysML Requirements in a package.

Trace: create Trace / DeriveReqt / Refine relations from your model requirements to the FC requirements.

Export: File → Export To → ReqIF, including the FC package and your requirements with their relations.

Import that file into Safety Lab (“📂 Import ReqIF”) — relations touching the FC objects become ⛓ traces.

Note — Requirement-to-requirement relations round-trip cleanly through ReqIF. Satisfy links from Blocks to requirements are a SysML concept that does not always survive ReqIF export — keep the round-trip traces at requirement level, and keep Block-level satisfaction inside the model.

# Lane 2 — Architecture via SysML XMI

In Cameo: File → Export To → Eclipse UML2 XMI (or native XMI).

In Safety Lab: use the AI Inputs importer and attach the .xmi file.

Blocks become systems, ports and connectors become interfaces, and SysML Requirements become requirement rows — each created with its trace at birth.

The imported architecture feeds the interdependence table, common-cause candidates, and MBSA coverage checks.

Note — A live Cameo bridge (direct XMI socket, no manual export) is roadmap item M5, pending a customer’s sample export to pin the exact vendor format. The manual XMI + ReqIF loop above works today.

# Suggested division of labor

| Artifact | Lives in | Travels by |
| --- | --- | --- |
| Failure conditions, FHA/PSSA/SSA, fault trees, bow-ties | Safety Lab | ReqIF export (FCs) |
| System architecture (blocks, ports, connectors) | Cameo | SysML XMI → Safety Lab |
| Requirements (system of record) | Jama / DOORS / Polarion | ReqIF or Jama live bridge |
| Trace assertions (requirement ↔ FC) | The RM tool, by the program’s engineers | Pulled back into Safety Lab |