Connecting Siemens Polarion to Safety Lab Aero

Traceability via ReqIF Round-Trip — No API Required

July 2026 · Safety Lab Aero

Table of Contents

Right-click and choose “Update Field” to build the table of contents.

# What this integration does

The posture in every integration is the same: failure conditions are authored and owned in Safety Lab; they are exported into the program’s requirements-management (RM) system as items; the program’s engineers assert the trace links there — in their system of record, under their signature; and Safety Lab pulls the relationship graph back. Traces then appear on the FHA rows (⛓ chips) and feed bow-tie barrier evidence at “tier 3” — derived from the program’s own trace data, not from text matching. The only manual step is the human act of tracing, which is the one step that should be manual.

Polarion’s transport is the ReqIF exchange file (Polarion supports ReqIF natively for both import and export, including .reqif and zipped .reqifz packages). The loop is: export the FC package from Safety Lab → import into Polarion → trace → export from Polarion → import back into Safety Lab. It works with no network connection between the tools — suitable for restricted environments.

# Step 1 — Export the FC package from Safety Lab

AC FHA page → Requirements traceability panel → “📄 Export ReqIF”. This downloads SafetyLab_AFHA_FCs_<date>.reqif — one “Failure Condition” object per FC (ForeignID = FC id, text, severity, flight phases) in a single specification hierarchy.

# Step 2 — Import into Polarion

In Polarion, expand Documents & Pages in Navigation and select the space (Index) that should hold the FC document.

Click Create New → ReqIF Document, and choose the exported .reqif file.

Map the incoming “Failure Condition” object type to a Polarion work-item type (e.g. a custom Hazard type or a generic requirement type), and map the fields (ForeignID, Text, Severity, Flight Phases).

Finish the import — the FCs appear as work items in a new Polarion Document.

Note — Your Polarion administrator may need to configure the ReqIF import mapping once; after that, re-imports reuse it.

# Step 3 — Trace (in Polarion — the human step)

Engineers link their requirement work items to the FC work items using a link role (e.g. relates_to, or a custom “mitigates” role). Standard Polarion linking — nothing special required.

# Step 4 — Export back from Polarion

Open the FC Document → ReqIF Export.

Include the linked work items and link roles in the export configuration (this is what carries the traces).

Export as .reqif (if your Polarion produces .reqifz, unzip it and use the inner .reqif file).

# Step 5 — Import into Safety Lab

AC FHA page → “📂 Import ReqIF” → choose the exported file. Relations touching the FC objects become ⛓ traces on the FHA rows and tier-3 evidence in bow-ties. FCs are recognized by their identifiers first and by ForeignID (the FC id) as a fallback, so the loop survives tools that rewrite identifiers. Re-imports de-duplicate by document key.

# Cadence and notes

When the FHA changes, export a fresh FC package; Polarion’s ReqIF re-import updates the existing document.

The FHA panel shows “📄 ReqIF” on exchanged FCs and the age of the last import.

A live Polarion REST adapter is on the Safety Lab roadmap — the ReqIF loop covers all Polarion versions today.